Section 01400 - QUALITY CONTROL

1.  TESTING SERVICES.  All tests to determine compliance with the Contract Documents shall be performed by an independent commercial testing firm acceptable to Owner.  The testing firm's laboratory shall be staffed with experienced technicians, properly equipped and fully qualified to perform the tests in accordance with the specified standards.

Testing services provided by Owner are for the sole benefit of Owner; however, test results shall be available to Contractor.  Testing necessary to satisfy Contractor's internal quality control procedures shall be the sole responsibility of Contractor.

1.01.  Testing Services Furnished by Contractor.  Unless otherwise specified, Contractor shall provide all testing services in connection with the following:

Concrete materials and mix designs.

Asphaltic concrete materials and mix designs.

Embedment, fill, and backfill materials.

All other tests and engineering data required for Owner's review of materials and equipment proposed to be used in the Work.

Contractor shall obtain Owner's acceptance of the testing firm before having services performed, and shall pay all costs for these testing services.

1.02.  Testing Services Furnished by Owner.  Unless otherwise specified, Owner shall provide for tests made on the following materials and equipment:

Concrete.

Asphaltic concrete.

Moisture-density and relative density tests on embedment, fill, and backfill materials.

In-place field density tests on embedments, fills, and backfill.

Other materials and equipment at the discretion of Owner.

Testing, including sampling, will be performed by Owner or the testing firm's laboratory personnel, in the general manner indicated in the Specifications.  Owner shall determine the exact time, location, and number of tests, including samples.

Arrangements for delivery of samples and test specimens to the testing firm's laboratory will be made by Owner.  The testing firm's laboratory shall perform all laboratory tests within a reasonable time consistent with the specified standards and shall furnish a written report of each test.

Contractor shall furnish all sample materials and cooperate in the testing activities, including sampling.  Contractor shall interrupt the Work when necessary to allow testing, including sampling, to be performed.  Contractor shall have no claim for an increase in Contract Price or Contract Times due to such interruption.  When testing activities, including sampling, are performed in the field by Owner or the testing firm's laboratory personnel, Contractor shall furnish personnel and facilities to assist in the activities.

If initial tests performed by Owner fail to meet the requirements of the specifications, Contractor shall reimburse Owner for all costs associated with retesting.

Contractor shall reimburse Owner for all testing services which are not properly scheduled.

1.03.  Transmittal of Test Reports.  Written reports of tests and engineering data furnished by Contractor for Owner's review of materials and equipment proposed to be used in the Work shall be submitted as specified for Shop Drawings.

End of Section